PHYS310 iLab Experiment 1

Equipment Checkout and PASCO Capstone Tutorial

Part 1: Equipment Checkout (required, but not graded)

Physics is an experimental science. Physical concepts are only held to be conditionally true as long as they are supported by experiment or observation. The beginning of modern physics can be traced back to Galileo. Galileo systematically used experiments to guide and test his theoretical ideas. He proved experimentally that Aristotle’s hypothesis on free fall (heavy objects fall faster than the light ones) is wrong. He showed that two objects of different masses that are simultaneously dropped from the top of Pisa’s tower hit the ground at the same time. He then discovered the law of free fall in a gravitational field by

studying the motion of objects rolling or sliding on the inclined plane and used these results to formulate the first principle of mechanics (later included by Newton in his Principia

Mathematica).

The laboratory is a workshop in which students can gain firsthand knowledge of the physical principles and experimental methods used in physics. For the student, some of the objectives of a laboratory exercise are to

	train in scientific methods of observation;

	acquire experience handling scientific and other electronics equipment;

	collect scientific data; and

	compute reliable results and draw valid conclusions from data.

For these reasons, all physics students should have the opportunity to participate in some form of laboratory experiment or exercise.

At the start of the course, you should order the required equipment kit as soon as possible. During Week 1, you should receive your kit and check the contents against this packing list. See PHYS310 Lab Equipment and Supplies. Report any discrepancies to your instructor. Please remember to read through the laboratory exercises well in advance of the due date to make sure that you have everything that you need.

Required Equipment:

PS-3200 PASCO AirLink: This component is used to interface PASCO PASPORT sensors with your computer to allow real-time data acquisition.

The AirLink is shown in Figure 1.

PS-2103A PASCO Motion Sensor: This component is used with the AirLink and PASCO Capstone to measure distance, velocity, and acceleration.

The motion sensor is shown in Figure 2.

PS-2189 PASCO High Resolution Force Sensor: This component is used with the AirLink and PASCO Capstone to measure force.

This sensor is shown in Figure 3.

Figure 1: PASCO AirLink

Figure 2: PASCO Motion

Sensor

Figure 3: PASCO High

Resolution Force Sensor

DeVry PHYS310 Laboratory Supplies Kit, eScience Labs

This kit (KIT2533) contains the balance of the equipment and supplies needed to complete the laboratory portion of PHYS310.

The contents of the kit are listed here.

| 1 | Caliper |
| --- | --- |
| 1 | Fan Cart |
| 2 | AA Batteries |
| 1 | Mass Set |
| 1 | Spool Thread |
| 1 | Fishing Line, 10 ft. |
| 1 | Polypropylene Graduated Cylinder (250 ml) |
| 1 | Table Clamp Pulley |
| 1 | S-clamp |
| 1 | #8 Rubber Stopper With Hole |
| 1 | #6 Rubber Stopper With Hole |
| 1 | Safety Glasses |
| 1 | PVC or Aluminum Tube 0.5 in. o.d. 12 in. Long |
| 2 | Plastic Beaker (250 ml ) |
| 2 | Styrofoam Cup |
| 2 | Plastic Cup |
| 2 | Paper Cup |
| 1 | Wooden Ruler, 12 inch With cm Marking and Central Groove |
| 5 | Marbles |
| 1 | Hardware Bag (including 6 hex nuts, 2 bolts, 15 washers, 3 fishing sinkers, 1 large hex nut) |
| 1 | Cardboard Ramp |

Part 2: PASCO Capstone Installation (required but not graded)

PASCO Capstone is the software that will allow you to acquire, analyze, and present data from the experiments in this course. Download and install PASCO Capstone from the Student Software Store by following the instructions provided in the last section of the lab

Part 3: PASCO Capstone Tutorial (graded) Objective

The objective of this exercise is to become familiar with some of the data analysis tools of the

PASCO Capstone software.

Parts and Equipment Required

	Computer with PASCO Capstone installed

Introduction

Computer acquisition and analysis of experimental data offers many advantages. Data are measured automatically and with greater precision than data measured by hand. The data are immediately stored in an electronic format and are available to be quickly graphed. There are also a wide variety of computerized data analysis tools available. PASCO Capstone is the software that we will use in this course to acquire and analyze experimental data. The purpose of this tutorial is for you to become familiar with PASCO Capstone and some of its tools.

Procedure

	Start PASCO Capstone by clicking on the program name in the Start menu.

	When PASCO Capstone opens, click on the Calculator button on the left side of the screen to expand the calculator tool panel as shown in Figure 4.

Figure 4: Expanding the Calculator Tool Panel

	Type the equation y = exp(-x) into the first calculator expression line (Figure 5) and click Accept.

	The calculator will identify x as a variable and show its definition “x =” on the next expression line. Right click on this line and select Insert Model Range; set the range from 0 to 10 in 100 steps as shown in Figure 5.

Figure 5: Creating a Model Range

	Click the Calculator button again to collapse the calculator tool panel, and then double-click the graph button on the right side of the screen to create a blank graph display.

	Click the Select Measurement button along the graph’s y-axis and choose the “y” function from the list under Equations/Constants. The curve representing the 
y = exp(-x) function will appear on the graph.

	Copy the graph from PASCO Capstone by making the graph the active window and selecting Copy from the Edit Menu.

	Paste the graph into the datasheet (PHYS310_W1_lab1_datasheet.docx) at the appropriate location by selecting Paste Special from the paste menu and then selecting Device Independent Bitmap. The graph should appear on the datasheet and can then be formatted and resized if you choose.

	Activate the graph’s Coordinates Tool shown in Figure 6, and drag the tool onto the graph to find the coordinates of the curve at the point x = 0.5

	Activate the graph’s Slope Tool shown in Figure 6, and drag the tool onto the graph to find the slope of the curve at the point x = 2.0.

	Record the results to answer question 2 on your datasheet.

Figure 6: Activating the Coordinates Tool and Slope Tool

	Click the Add Page button  above the Tools Panel, and then select the Table & Graph template in the new blank page as shown in Figure 7.

Figure 7: New Page with Table and Graph

	Type the values from Table 1 below into the blank columns of the table.

	When you begin typing into the columns, the table will automatically name the columns User Data 1 and User Data 2. Click on each column name and rename them to X and Y (both capital letters), like the table below.

	In the graph, click the Select Measurement buttons along both axes and choose Y for the y-axis and X for the x-axis.

	Click the curve fit drop-down button in the graph and select Linear from the list to find the best straight line fit to the data.

	Record the values of the slope and the y-intercept on your datasheet.

	Save your datasheet.

	Turn the datasheet in to your instructor.

Table 1: Sample Data for Graph

| X | Y |
| --- | --- |
| -1 | 0.43 |
| -0.5 | 1.435 |
| 0 | 2.38 |
| 0.5 | 3.435 |
| 1 | 4.32 |
| 1.5 | 5.355 |
| 2 | 6.51 |
| 2.5 | 7.445 |
| 3 | 8.44 |
| 3.5 | 9.455 |
| 4 | 10.43 |
| 4.5 | 11.455 |
| 5 | 12.44 |

Installing Capstone

Pasco Capstone volume license key:  18jkm-5u0t7-afol1-j58ob-20qr6-c8933

To access PASCO Capstone, under Course Home, click on Course Resources and enter the Student Software Store

Click the Student Software Store button and a new window will pop up.  Scroll down to Go To the Student Software Store and click

In the pop-up window, click on Course Related Software

Click on Pasco Capstone

Add to Cart

Checkout through the shopping cart and download your software

Download the SDX manager then download then download the .SDX file

Open the .SDX file and Launch

Capstone is now fully loaded.

By double clicking on a Capstone file (*.cap), Capstone will automatically open

You can also open a file by clicking on the Open Experiment icon circled above.