# EET-350

Signal Processing with lab

Lab #6

Dr.  Omar

Lab#6

# IIR Digital Filter Design using Bilinear Transformation

Objective:

- To learn about how to design IIR filters using Matlab’s bilinear transform command.

Introduction

An IIR filter is completely specified by its difference equation or transfer function

As given in general forms in the following

Where  and  are coefficients of the system;  is the discrete-time index. A digital filter can also be represented by its transfer function by taking the z-transform of the difference equation as

The IIR filter output  depends not only on the current input  and past inputs but also on the past outputs  (feedback or recursive terms). The transfer function of IIR filters is a rational function of  (i.e., numerator polynomial on denominator polynomial in  . Consequently the filter impulse response has an infinite number of terms.

The roots of the denominator polynomial obtained by solving characteristic equation  are called poles and must be inside the unit circle in z-plane to ensure the filter stability.

IIR filters offer a much smaller filter size. Hence, the filter operation as coded on the digital computers or DSP chips requires a small number of computations. On the other side, the linear phase response is hard to achieve. When linear phase is not necessary, IIR filters are preferred due to simpler computation and faster response as compared to an FIR filter with a large number of coefficients or taps.

Part I: Design a second order IIR Butterworth low pass digital filter with a cutoff frequency of 500 Hz and a sampling frequency of 10,000 Hz using bilinear transformation then find the following:

- The output (response) due to the following inputs:

    - Sinusoidal signal with a frequency of 100Hz.

    - Sinusoidal signal with a frequency of 500Hz.

    - Sinusoidal signal with a frequency of 2000Hz.

- Repeat (a) above for a 6th order Butterworth filter

Part II: Repeat Part I using IIR Chebyshev digital filter with a passband ripple of 3 dB.

Helpful Notes

Sample partial code

[z,p,n]=buttap(2);  %create a 2-pole Butterworth filter

% convert(z,p,n) format to a transfer function format

[num,den] = zp2tf(z,p,n);

% need to transform the cutoff frequency to 500 Hz

wc= 2* pi * 500 ;

[num,den] = lp2lp(num,den,wc);		% (the Bs and As for the 500Hz cutoff freq.)

T = 1/ 10000.0 ; 				% sampling frequency = 10,000 Hz

[numd,dend]= bilinear(num,den,1/T);	%(the Bs and As for the LPF digital filter.)

figure(1)

n=0:80;

x1=sin(2*pi*200* n*T);

x2=sin(2*pi*500* n*T);

x3=sin(2*pi*2000* n*T);

y1=filter(numd,dend,x1);	%filter output due to input signal x1

y2=filter(numd,dend,x2); 	%filter output due to input signal x2

y3=filter(numd,dend,x3); 	%filter output due to input signal x3

This lab’s Matlab functions and their use are:

BUTTAP :  (Butterworth analog lowpass filter prototype).

[Z,P,K] = BUTTAP(N) returns the zeros, poles, and gain for an N-th order normalized prototype Butterworth analog lowpass filter.  The resulting filter has N poles around the unit circle in the left half plane, and no zeros.

ZP2TF  (Zero-pole to transfer function conversion.)

[NUM,DEN] = ZP2TF(Z,P,K)  forms the transfer function:

H(s)=NUM(s)DEN(s)

given a set of zero locations in vector Z, a set of pole locations in vector P, and a gain in scalar K.  Vectors NUM and DEN are returned with numerator and denominator coefficients in descending

powers of s.

LP2LP (Lowpass to lowpass analog filter transformation.)

[NUMT,DENT] = LP2LP(NUM,DEN,Wo) transforms the lowpass filter prototype NUM(s)/DEN(s) with unity cutoff frequency of 1 rad/sec to a lowpass filter with cutoff frequency Wo (rad/sec).

BILINEAR   (Bilinear transformation with optional frequency prewarping.)

[NUMd,DENd] = BILINEAR(NUM,DEN,Fs), where NUM and DEN are row vectors containing numerator and denominator transfer function coefficients, NUM(s)/DEN(s), in descending powers of

s, transforms to z-transform coefficients NUMd(z)/DENd(z).

where Bilinear transformation is given by:

H(z) = H(s) 	|

| s = 2*Fs*(z-1)/(z+1)

Where Fs is the sampling frequency in Hz (Fs = 1/T)

To find the response y1, due to an input signal x1 use:

y1 = filter(num,den,x1)

This filters the data in vector x1 with the filter described by vectors num and den to create the filtered data y1.